DEPTH RESOLUTION CALCULATION

for targets containing multilayers

by E. Szilágyi

Using this program you can calculate:

(1) the depth resolution for Elastic Recoil Detection Analysis (ERD), Backscattering Spectrometry (BS), Forward Scattering (FS) and Nuclear Reaction Analysis (NRA) for a given geometry;

(2) the optimal depth resolution;

(3) the energy loss and the Bohr straggling of the incident ion (at the incident energy) going through the absorber foil;

(4) the Tschalar effect without and with a given inhomogeneity;

(5) sub-spectra accounting for all energy-spread effects of RBS and ERDA

In calculations of the energy-spread values for the energy loss methods we take into account the following contributions (in FWHM sense) [1]:

– The energy and angular spread of the beam. The two are related to the energy stability of the accelerator and to the beam handling system, respectively.

– The geometrical spread caused by the finite beam spot size and detector solid angle.

– The straggling of incident ions and emitted particles in the target. Here both the nuclear and electronic straggling contributions are treated. The latter can be calculated both with and without Chu's correction. The charge state fluctuations for heavy ions are treated using Yang’s formulae. We also take into account the effect of change in stopping power for large energy losses. We assume the straggling contributions to be Gaussian-shaped and the particles' paths in and out produce independent effects.

– The multiple scattering of incident ions and emitted particles in the target [2], where only projected distributions play a role. The multiple scattering causes energy spread in two ways:

(i) the energy of the ions changes because of the different path lengths, and

(ii) the energy of the emitted particle depends on the reaction angle.

The combination of the last two effects is treated as statistically dependent, while the ways in and out are independent. The shape of the resulting energy spread was approximated with Pearson VII distribution (The Gaussian and the Breit - Wigner distributions are special cases derived from Pearson VII.) Applying a special procedure, we are able to treat multi-elemental samples and large values of energy losses as well.

– The energy resolution of the detecting method, it is taken into account with Gaussian shape.

– The absorber foil (if there is any). The width of the detected energy spread measured with an absorber foil is obtained in two steps. First we calculate the width and shape of the energy distribution just at the exit from the sample, then using the transfer function of the foil we determine the correct detected energy spectrum and its FWHM using numerical integration.

Numerical convolution of the above mentioned independent contributions would be carried out to obtain the total energy distribution before the absorber foil. This would be, however, a relatively slow procedure. To avoid this, an approximative fast summation method was worked out for Pearson VII distribution.

The following effects are not included in our calculations: plural scattering, extremely small energy losses, non-Gaussian straggling for small depths and sample surface roughness.

The mdepth.zip file contains the following directories and files:

| Directory | file name | Remarks |
| --- | --- | --- |
| c:\mdepth\ | mdepth.exe default.par inp.asc readme.doc | the code default parameter file sample input file (Word for Windows 6.0), this file |
| c:\mdepth\lev\ | chupar.dat fmt.dat foils.dat reactions.dat stopping.dat taufi.dat taufij.dat *.lev f??.dat | Fitted parameter values for Chu’s correction Fitted parameters for convolution of Pearson VII function Identify the different, already calculated absorber foils Q value of the reactions for the ground state Stopping power data for multiple scattering calculation for multiple scattering calculation level structures for different nuclei: e.g. O16.lev energy straggling of the absorber foils |
| c:\mdepth\par | *.par | directory for the parameter files |
| c:\mdepth\res | *.dat | directory for the result files |
| c:\mdepth\spe | *.dat | directory for calculated sub spectra (ASCII) |

How to use the program:

To start the program type mdepth. When the program asks where your parameter file is enter the full directory path e.g.,: c:\depth\par\erdc.par or press ENTER (in this case the program will start using the default parameter file).

Next, you can change the path to directories for level, parameter, result and spectrum files. To select use the letters and ENTER.

Warnings:

a) always end path name with a backslash. e.g. c:\user\res\

b) the result and the spectrum directories have to be different.

After setting the paths, press N and ENTER.

Next, the program reads the parameters and the stopping power data. After this step you can change parameters. Generally, a letter is assigned to each functionality of the program (select an option or set a parameter). In order to execute a particular functionality you must type the letter followed by ENTER. The program shows the actual values of the most important parameters. To modify them enter the appropriate letters and the value of the parameter to be changed. E.g.,:

– if you want to load (or save) a parameter set, press L (S) (and ENTER), then type filename;

– to change the scattering angle press G (and ENTER), then T (and ENTER) and type the new value (and ENTER).

– to change the target material press T (and ENTER), then the number of the elemental component to be changed, the element’s symbol, then its abundance (and ENTER). To erase a component instead of its symbol and abundance type 0.

If there is no absorber foil then set its thickness to 0, but set at least one elemental component. For the depth resolution calculation it is necessary to give the density of the target material in [g/cm3] or [1022 at/cm3] units.

Detector position:

IBM – if the detector is mounted in the plane determined by the beam and the sample surface normal

KFKI – a special case of the IBM, when the incoming angle is constant. We use 10.5 [deg].

CORNELL – if the detector is mounted below the plane determined by the beam and the sample surface normal.

The energy resolution of the detector, dEdet is fitted using the following function:

dEdet = a0 + a1 Ea2 + a3 Ea4

In the depth resolution calculation a rough error estimation is given. If the theory described below is not valid anymore the program gives a total depth resolution equal to 10000 nm. The program gives the maximal probing depth, when the mean detected energy of the recoils will be smaller than the threshold energy. For the depth resolution calculation you have to give the target element and its mass number specified in the reaction part, its abundance is used only for the spectrum calculation. For the elastic scattering the Qm value and the excitation level should be equal to 0.

To set the energy calibration you have two possibilities: You can set it manually or let the program calculate it using a linear fit. In latter case you need at least two points given by different reactions or geometries.

* Warning!

The parameter file does not contain the necessary values for the fit, so if you use a new set of parameters, you cannot continue the energy calibration.

Before starting your calculation check all the parameters carefully.

When the program finishes, you can save the results by entering the results file name. If you don't want to save it, press ENTER.

When the calculation done, the program may give the following messages:

– E < dE/10: the detected mean energy is smaller than the tenth of the detected energy spread

– absorber: the FWHM of the detected energy spread cannot be evaluated because it is too close to zero.

– detector resolution reaches 0.

– ctg : if somewhere: abs(/2 ctg )>1 meaning that the approximation used is not valid any more

– error>500%: meaning that the approximation used is not valid any more.

– stopping power = 0

– sample thickness!!! : The ions reach the end of the sample.

– Max. depth = ..... : The ions reach the maximal depth value.

– e < thr : the energy of the ions reaches the threshold energy

– the number of sublayers reaches the limit (150)

(1) Depth resolution of IBA methods

Theory of the depth resolution calculation (see ref. 1 for details).

Error calculation: The whole error of the calculation is not given. We estimate only the error (given in %) due of our approximations, deduced from the geometry and multiple scattering energy spread contributions:

1cosβ+sinβctgα»1−βctgα

where in the case of the multiple scattering  is the incoming (or outcoming) angle,  is the cord angle of the multiple scattering inward (or outward). In the case of the geometry contribution  is the outcoming angle,  is the projected effective detector acceptance angle.

Output format

After the calculation the results can be saved in two files:

a) the first one contains the following data:

/*	REFLECTION GEOMETRY

/*	theta = 165.00       alpha = 20.00

/* depth|depth & energy|   before the    |         energy spread contributions                    | error | mean

/*          |  resolution      |    reaction      |            after the sample                                  | from  | det.

/*   x     |  dx     det        |  E     dE    dfi | beam  ang  geo    ss     msi    mso  doppl  tot  | appr. |   E

/* [nm] | [nm]   [keV]   |   [keV]    [deg]|                      [keV]                                       |   %    | [keV]

0      30.63   16.4      1620    1   0.2    0.6    0.2    3.3    0.0    0.0    0.0    0.1    3.4    0.0    920.2

4      30.72   16.4      1619    1   0.2    0.6    0.2    3.3    1.3    0.0    0.0    0.1    3.6    0.0    918.0

14       7.89   17.2      1606    4   0.3    0.6    0.2    3.8    5.1    0.3    0.5    0.1    6.4    0.0    896.2

column numbers:

1          2        3          4       5     6      7        8       9     10      11     12     13     14     15      16

The meaning of the columns are, as follows:

1)	the depth in nm units,

2)	the depth resolution in nm units,

3)	the energy resolution of the detected ions in keV,

4)	the mean energy of the incident ions before the reaction [keV],

5)	the energy spread of the incident ions before the reaction [keV],

6)	the angular spread of the incident ions before the reaction [deg],

Then the calculated energy spread contributions are listed, just after the ions leave the sample (before the detector and absorber foil) taking into account all the necessary corrections.

7)	energy spread of the beam [keV],

8)	energy spread caused by the angular spread of the beam [keV],

9)	geometrical energy spread [keV],

10)	straggling in the sample (inward and outward) [keV],

11)	energy spread caused by the multiple scattering effects (lateral and angular), inward [keV],

12)	energy spread caused by the multiple scattering effects (lateral and angular), outward [keV],

13)	energy spread caused by the doppler effect [keV],

14)	the “sum” of these contributions [keV],

Then:

15)	error coming from the main approximation [%],

16)	the mean detected energy, [keV].

b) The second file contains some detailed results mainly connected to the multiple scattering calculations.

alpha in = 20.0    alpha out = 20.0   theta  = 165.0

Z1 =  2    M1 =  4    Z2 =  2    M2 =  4

depth| straggling| ms in after the sample|   ms out after the sample   |S eff

| in  | out   |  tau |  fi  |   nu  |  k       |  tau |  fi   | nu  |    k   |    k  |

[nm] |   [keV]    |   -   |[deg]|   -   |   -       |   -   |[deg]|  -   |   -    |    -   [keV/nm]

0      0.0   0.0   0.0  0.00  2.00   0.0        0.0  0.00  2.00   0.0   0.0   0.53

4      1.1   1.2   0.6  0.02  1.06   0.0        0.7  0.04  1.06   0.0  -0.0   0.53

14     4.4   4.4   2.0  0.25  1.25   0.0        2.0  0.43  1.24   0.0  -0.1   2.18

Column number:

1       2       3     4      5       6       7           8      9      10    11    12      13

1)	the depth in nm units,

2)	straggling inward [keV],

3)	straggling outward [keV],

the multiple scattering data for the inward path: for details see ref. 1

4)	tau - the reduced path length,

5)	fi - FWHM of the angular spread,

6)	nu(tau) - see: eq. 45 in ref. 1. ,

7)	k-perpendicular (the ratio of the lateral and angular spreads),

the multiple scattering data for the outward path

8)	tau - the reduced path length,

9)	fi,

10)	nu,

11)	k-perpendicular,

12)	k-parallel,

13)	effective stopping power [keV/nm].

(2) Optimal depth resolution

The optimal depth resolution as a function of the depth can be calculated using different geometries. If the detector position is IBM or CORNELL the program changes automatically the angles with a given step between minimal and maximal values of the scattering and incoming angles. If the detector angle is fixed you can give 4 different depths, for which the program saves the depth resolution values as a function of the incoming angle. In this case you have to give depth values divisible by the step given.

In the case of the KFKI detector position the outcoming angle is fixed.

Output format

a) optimalization: (example for ERD: Theta min = 10 Theta max = 20 ; alpha min =5; step 5 degrees)

/*                        optimal values

/*  depth     depth res.     alpha       theta        error

/*  [nm]         [nm]         [deg]      [deg]         [%]

0         95.783       5.000      10.000        0.00

10        100.524       5.000      10.000       -0.00

20         14.914       5.000      10.000       -0.00

The first column here is the depth in nm, the second one is the depth resolution in nm, the next two column the geometry (alpha and theta, both in degrees) and the last one the error value.

b) at fixed scattering angle: (example for ERD: Theta min = Theta max = 20 ; alpha min =5; step 5 degrees) In this case you can save the results into two files; the first one contains the optimal depth resolution as a function of depth (and the geometry). (Similar file as the full optimalization, except here theta is fixed.)

/*                        optimal values

/*  depth     depth res.     alpha       theta        error

/*  [nm]         [nm]         [deg]      [deg]         [%]

0        159.856       5.000      20.000        0.00

10        154.875       5.000      20.000       -0.00

20         23.770       5.000      20.000       -0.00

The first column here is the depth in nm, the second one is the depth resolution in nm, the next two column the geometry (alpha and theta, both in degrees) and the last one the error value.

The second file contains the depth resolution values at the selected depth:

/*   alpha        theta      depth resolution at x =

/*   [deg]        [deg]        0.0          10.0          20.0          30.0 [nm]

/*

5.000       20.000      159.856      154.875       23.770     25.711

10.000       20.000      220.507      220.821       34.098     37.861

15.000       20.000      225.764      236.336       34.735     40.210

The first two columns contain the geometry (alpha and theta) then the next 4 ones the depth resolution values (at the surface, 10, 20 and 30 nm).

Warning! Be careful with the layer structure and the iteration step. If the depth resolution is not calculated in the selected depth, this option will not work properly.

(3) Energy loss of the incident ion (at the incident energy) going through the absorber foil and its Bohr straggling

This feature is useful to calculate the thickness of the absorber foil for ERDA or calibration of NRA spectra, (the result can be saved).

Output format

/*  depth   Energy     Stopping power

/* [micron]  [keV]      [keV/micron]

0.0000   1620.0000   228.3863

0.0500   1608.5807   229.1652

0.1000   1597.1224   229.9507

0.1500   1585.6249   230.7430

0.2000   1574.0877   231.5420

0.2500   1562.5106   232.3477

The first column here is the depth in micron, the second one is the energy in keV and third one is the stopping power in keV/micron.

(4) Tschalar effect with and without a given inhomogeneity;

The energy straggling of the absorber foil can be calculated taking into account the effect of change in stopping power for large energy losses. This option can be used to estimate inhomogeneity of the absorber foil (see ref. 1). (Similar calculation is made automatically for the absorbers.)

The straggling values are calculated in two energy ranges with given energy steps.

Output format

e.g.: for 24 micron Mylar foil: E1 = 6000 keV, step1 = 250 keV and E2= 2000 , step2=50 we get:

/*  E             rel.E    de (without) de (with)

/*                              inhomogeneity

/* [keV]       -            [keV]         [keV]

6000.000      0.037     39.559     40.657

5750.000      0.040     39.620     40.797

5500.000      0.043     39.689     40.955

5250.000      0.047     39.767     41.133

5000.000      0.052     39.856     41.335

4750.000      0.057     39.958     41.567

4500.000      0.063     40.077     41.835

4250.000      0.069     40.216     42.146

4000.000      0.077     40.380     42.512

3750.000      0.087     40.575     42.946

3500.000      0.099     40.812     43.469

3250.000      0.113     41.103     44.108

3000.000      0.131     41.469     44.905

2750.000      0.154     41.939     45.923

2500.000      0.183     42.567     47.266

2000.000      0.279     44.785     51.873

1950.000      0.293     45.143     52.593

1900.000      0.308     45.543     53.392

1850.000      0.324     45.996     54.287

1800.000      0.342     46.512     55.295

1750.000      0.362     47.106     56.443

1700.000      0.383     47.800     57.765

1650.000      0.407     48.622     59.306

1600.000      0.434     49.614     61.134

1550.000      0.463     50.836     63.343

1500.000      0.497     52.386     66.084

1450.000      0.535     54.418     69.597

1400.000      0.580     57.210     74.302

1350.000      0.633     61.288     80.994

1300.000      0.697     67.777     91.366

The first column here is the incident energy in keV, the second one is the relative energy loss – the ion energy after the foil / incident energy and the last two column contain the straggling in the foil without and with inhomogeneity in keV.

(5) sub-spectra accounting for all energy-spread effects of RBS and ERDA

During the depth resolution calculation the program derives also a sub-spectrum. For He-ERDA a special cross section fit was applied (see ref. 2 for details) For RBS and heavy ion ERDA only the Rutherford cross section is used. All the other cases (FS, NRA) the cross sections are missing (d/d=1).

Output format

/* energy [keV]     yield

120.73              0.00

124.42            360.34

128.11            719.74

131.80           1047.08

135.49           1256.83

139.18           1355.81

The first column here the detected energy, the second one is the yield.

References:

[1] 	E. Szilágyi, F. Pászti and G. Amsel: 
Theoretical approximations for depth resolution calculations in IBA methods, 
Nucl. Instr. Meth. B 100 (1995) 103.

[2] 	G. Amsel, G. Battistig and A. L'Hoir:
Small angle multiple scattering of fast ions, physics, stochastic theory and numerical calculations,
Nucl. Instr. Meth. B 201(2003)325-388

[3] 	E. Szilágyi, L.S. Wielunski and F. Pászti: 
Theoretical approximations of energy distributions of elastic recoiled hydrogen atoms, 
Nucl. Instr. Meth. B 136-138(1998)701-706.

Other related papers:

[a] 	F. Pászti, E. Szilágyi and E. Kótai: 
Optimization of the depth resolution in elastic recoil detection, 
Nucl. Instr. Meth. B54 (1991) 507.

[b] 	E. Szilágyi, F. Pászti, V. Quillet and F. Abel: 
Optimization of the Depth Resolution in ERDA of H using 12C ions, 
Nucl. Instr. Meth. B85 (1994) 63.

[c] 	E. Szilágyi and F. Pászti: 
Theoretical calculation of the depth resolution of IBA methods, 
Nucl. Instr. Meth. B85 (1994) 616.

[d] 	N.P. Barradas, J.C. Soares, M. F. da Silva, F. Pászti and E. Szilágyi: 
Study of multilayer substrate surface roughness using RBS with improved depth resolution, 
Nucl. Instr. Meth. B 94 (1994) 266.

[e] 	L.S. Wielunski, E. Szilágyi and G.L. Harding: 
Multiple scattering effects in the depth resolution of elastic recoil detection, 
Nucl. Instr. Meth. B 136-138(1998)713-718.

[f]	E. Szilágyi: 
Energy spread in ion beam analysis,
Nucl. Instr. Meth. B 161-163(2000)37-47.

[g]	E. Szilágyi:
On the limitations introduced by energy spread in elastic recoil detection analysis,
Nucl. Instr. Meth. B 183(2001)25-33.

[h]	L.S. Wielunski, D. Grambole, U. Kreissig, R. Grötzschel, G. Harding, E. Szilágyi:
Hydrogen depth resolution in multilayer metal structures, comparison of elastic recoil detection and resonant nuclear reaction method,
Nucl. Instr. Meth. B 190(2002)693-698.

If you achieve publishable results with the help of this program please reference to publication [1].

If you have any problem or question using this program please feel free to contact me at:

Edit Szilágyi

MTA KFKI Research Institute for Particle and Nuclear Physics

H - 1525 Budapest, P.O.Box 49, Hungary

Phone:	+36-1-392-2222/39-62

Fax : 	+36-1-395-9151

Email:	SZILAGYI@RMKI.KFKI.HU